OPERATOR USER MANUAL

# Purpose of the System

- The Pneumatic Laser QC System automatically inspects laser welding depth during production, determines PASS / FAIL results, and generates QR-based traceability for accepted parts.

- The system is designed for continuous shop-floor operation with minimal operator intervention.

# Operator Login & Access

- System runs in kiosk mode

- No login required for normal operation

- Operators cannot access settings

- Critical actions are password-protected

# Main Operator Screen (Dashboard)

Figure 1: Main Dashboard

## What the Operator Sees

- Live laser waveform graph

- Latest inspection result (PASS / FAIL)

- QR code (PASS only)

- Recent production cycles

- System health indicators (PLC, Printer, Modem)

## Operator Responsibilities

- Monitor PASS / FAIL status

- Ensure printer is online

- Respond to FAIL conditions as per SOP

- Inform supervisor if system error appears

# Understanding PASS / FAIL Results

Figure 2: Result Panel - Pass

Figure 3: Result Panel - Fail

## PASS

- Weld depth within configured limits

- QR code generated automatically

- Part allowed to move to next process

## FAIL

- Weld depth outside limits

- No QR code generated

- Part must be segregate

# QR Label Printing (Operator)

Figure 4: Print icon in Header

Automatic Printing

- PASS parts are queued automatically

- Labels are printed without manual action

Figure 5: QR Label - RHD

Figure 6: QR Label - LHD

## Manual Printing (Pending Queue

- Open QR Printing

- Select “Pending QR Labels”

- Select required rows

- Click Print Selected

Figure 7: Pending QR Labels - Unselected

Confirm print

Figure 8: Pending QR Labels - 3 rows selected

Figure 9: Print Dialogue - Confirmation to print QR Labels

Figure 10: Search and Print QR Label Screen – Pre-Search

Manual QR Search & Reprint

## When to Use

- QR label damaged

- Printer failure during print

- Audit or verification request

Figure 11: Search and Print QR Label Screen - Post Search

## Steps

- Open Search & Print QR

- Enter exact QR text

- Click Search

- Verify preview details

- Click Print QR Label

Figure 12: Application Settings Icon in Header

Operator Access & Security

Operator access is protected by a settings password.

Protected functions include:

- Model management

- QR configuration

- Alert contact management

Figure 13: Prompt for entering the security password to access settings

System shutdown

Figure 14: Model Management screen

Model Management

Each model represents a product with:

- Welding tolerance limits

- Model type (RHD / LHD)

- QR behavior

Supervisor Actions

- Add new models

- Edit tolerance limits

- Activate one model at a time

- Delete inactive models only

✔ Only one model can be active
✔ Active model is clearly highlighted in green colour

Figure 15: Activate Model Screen

Model Activation & QR Rules

During Activation, Supervisor can:

- Set QR prefix

- Set starting QR counter

- Preview next QR label

✔ Prevents duplicate QR codes
✔ Ensures traceability consistency

Figure 17: Edit Existing Model Screen

Figure 16: Add New Model Screen

Add and Edit Models

Figure 18: Alert Contacts Screen

Alert Contact Management

Define who receives SMS alerts for quality/system events.

- Model-specific contacts

- Add / Edit / Delete contacts

- Phone number validation

## Password Management

Figure 19: Change Security Password Screen

Maintain confidentiality

- Change password periodically

- Ensure only authorized access

Safe Shutdown Procedure

- Authenticate with password

- Confirm shutdown warning

- Ensure no active cycle is running

- Exit application safely