# Purpose and Scope

This chapter defines the User Interface (UI) architecture of the Welding Quality Control System.

The UI architecture supports:

- Real-time monitoring of welding process parameters

- Objective evaluation of weld quality (PASS / FAIL)

- Traceable presentation of production data

- Controlled system operation in industrial and kiosk environments

# Design Principles

The UI architecture is developed based on the following principles:

- Process Transparency – All critical process data is visible in real time

- Error Prevention (Poka-Yoke) – QR generation and printing are restricted to PASS results only

- Traceability – Every weld cycle is associated with model, timestamp, and measured values

- Access Control – Configuration and shutdown actions are password-protected

- Deterministic Layout – Fixed layouts prevent operator confusion and misinterpretation

- Continuous Operation – Designed for 24×7 industrial use without manual intervention

# Overall UI Layout Architecture

The system UI is organized into five permanent functional zones:

- Application Header

- Real-Time Process Visualization Area

- Weld Result Evaluation Area

- Latest Cycles Summary Area

- System Footer

All zones remain consistently positioned across screens to ensure operator familiarity and audit consistency.

Application Header – System Identification & Control

The Application Header provides system identification, real-time clock reference, and controlled operator actions.

- Fixed-height industrial header (non-resizable)

- Company logo and system name

- Live date and time display (system clock)

- Icon-only action controls:

  - Print (QR label printing)

  - Settings (password-protected)

  - Shutdown (available only in kiosk mode)

Real-Time Process Visualization – Plot Panel

The Plot Panel provides continuous real-time visualization of laser welding signals.

Features:

- Live waveform of laser height versus time

- Fixed time window with automatic scaling

- Visual weld cycle annotations

- Clear PASS / FAIL indication with measured weld depth

- Display of model-specific acceptance limits

- Branding watermark for system identification

Weld Result Evaluation – Result Panel

The Result Panel presents the final quality decision for each weld cycle.

Features:

- Large PASS / FAIL display with color coding

- Weld depth measurement display

- Model name and model type

- Time-stamped cycle completion record

- QR code text display (PASS cycles only)

- Adaptive font sizing for variable-length QR data

Understanding Results Panel,

- PASS

  - Weld depth within configured limits

  - QR code generated automatically

  - Part allowed to move to next process

- FAIL

  - Weld depth outside limits

  - No QR code generated

  - Part must be segregate

## Latest Cycles Summary – Cycles Panel

The Cycles Panel provides a compact historical view of recent production cycles.

Features:

- Displays latest completed cycles (newest first)

- PASS / FAIL clearly highlighted per cycle

- Model name, weld depth, and timestamp

- QR code displayed only for PASS cycles

- Fixed-height, non-scrollable layout for stability

## System Footer – Status & Notification Panel

The Footer Panel provides continuous system health monitoring and notification feedback.

Features:

- Live status indication for:

  - PLC communication

  - GSM modem

  - USB printer

- SMS notification display with:

  - Recipient

  - Timestamp

  - Message content

  - Controlled scrolling for long messages

Automatic message timeout and clearance

# Main Application Control & Workflow

## System Behavior:

- Automatically loads the active welding model

- Continuously receives and displays laser data

- Updates all UI panels upon cycle completion

- Synchronizes plot, result, and cycle history

- Maintains layout consistency throughout operation

## Kiosk Mode Controls:

- Full-screen operation

- Cursor auto-hide after inactivity

- No accidental window switching

- Controlled shutdown sequence with confirmation

## Controlled Shutdown & Safety

- Shutdown Confirmation

- Password-protected access

- Explicit confirmation dialog

- Warning to ensure no active weld cycle

- Safe application termination

# QR Label Printing (Operator)

## Automatic Printing

- PASS parts are queued automatically

Labels are printed without manual action

Manual Printing (Pending Queue)

Open QR Printing

- Select “Pending QR Labels”

- Select required rows

- Click Print Selected

Confirm print

Manual QR Search & Reprint

When to Use

- QR label damaged

- Printer failure during print

- Audit or verification request

Steps

- Open Search & Print QR

- Enter exact QR text

- Click Search

- Verify preview details

Click Print QR Label

Operator Access & Security

Operator access is protected by a settings password.

Protected functions include:

- Model management

- QR configuration

- Alert contact management

System shutdown

## Model Management

Each model represents a product with:

- Welding tolerance limits

- Model type (RHD / LHD)

- QR behavior

Supervisor Actions

- Add new models

- Edit tolerance limits

- Activate one model at a time

- Delete inactive models only

✔ Only one model can be active
✔ Active model is clearly highlighted in green colour

Model Activation & QR Rules

During Activation, Supervisor can:

- Set QR prefix

- Set starting QR counter

- Preview next QR label

✔ Prevents duplicate QR codes
✔ Ensures traceability consistency

Add and Edit Models

Alert Contact Management

Define who receives SMS alerts for quality/system events.

- Model-specific contacts

- Add / Edit / Delete contacts

- Phone number validation

## Password Management

Maintain confidentiality

- Change password periodically

- Ensure only authorized access

Safe Shutdown Procedure

- Authenticate with password

- Confirm shutdown warning

- Ensure no active cycle is running

- Exit application safely